、、、、、、、、、、、

Design principles you can refer to

# Mathematical Forms to Electrical Components and Circuits Mapping

| Mathematical Form | Electronic Component | Circuit Combination Form |

|-------------------|----------------------|--------------------------|

| Constant (k) | Ideal voltage source | V = k |

| Linear relationship (y = mx) | Resistor | V = IR (Ohm's Law) |

| Exponential (e^x) | Diode | I = Is(e^(V/Vt) - 1) |

| Trigonometric (sin cos) | AC source | V = Vm * sin(ωt) |

| Integration (∫) | Capacitor | I = C * dV/dt |

| Differentiation (d/dt) | Inductor | V = L * dI/dt |

| Logarithmic (log) | Bipolar transistor | Ic = Is * e^(Vbe/Vt) |

| Square law (x^2) | MOSFET (in saturation) | Id = K(Vgs - Vth)^2 |

| Step function | Switch | Ideal switch: V = 0 (closed) I = 0 (open) |

| Piecewise linear | Zener diode | V = Vz for I > Iz else follows diode equation |

| Complex impedance (Z = R + jX) | RLC circuit | Z = R + j(ωL - 1/(ωC)) |

| Transfer function (H(s) = Y(s)/X(s)) | Op-amp based filter circuits | Examples: Low-pass: H(s) = 1/(1+sRC), High-pass: H(s) = sRC/(1+sRC), Band-pass: H(s) = (sR/L)/(s^2 + sR/L + 1/LC) |

| PID Control (U(s) = Kp + Ki/s + Kd s) | Op-amp circuits | Proportional: Simple amplifier, Integral: Op-amp integrator, Derivative: Op-amp differentiator, Combined: Full PID circuit |

| Multiplication (x * y) | Mixer or multiplier IC | Vout = k * V1 * V2 |

| Division (x / y) | Analog divider IC | Vout = k * (V1 / V2) |

| Square root (√x) | Analog computation circuit | Using op-amps and multipliers |

| Hyperbolic tangent (tanh) | Differential pair | Iout = Itanh(Vin / (2Vt)) |

| Summation (Σ) | Summing amplifier | Vout = -Rf * Σ(Vi/Ri) |

| Comparator function (sgn(x)) | Comparator IC | Vout = Vcc if V+ > V- else Vee |

| Delay (x(t-τ)) | Delay line or shift register | Digital: bucket-brigade device; Analog: transmission line |

| Hysteresis | Schmitt trigger | Output depends on input and previous state |

| Oscillation | LC tank circuit | f = 1 / (2π√(LC)) |

| Feedback system | Operational amplifier circuit | Various configurations (inverting non-inverting etc.) |

| Convolution | Matched filter | h(t) * x(t) implemented as a tapped delay line |

| Fourier transform | Spectrum analyzer | Converting time-domain to frequency-domain |

| State-space model | Active filter, Multivariable Feedback System (MIMO) | Used for representing higher-order systems or controlling multiple states |

| Root Locus | Analog computer, Programmable Analog Array (PAA) | Analysis of pole-zero placement as a function of gain |

| Nyquist Criterion | Feedback circuit with Oscilloscope | Stability analysis in frequency domain using Nyquist plot |

| Bode Plot | Oscillator, Variable Frequency Source | Magnitude and phase response analysis for tuning circuits |

| Model Predictive Control (MPC) | Digital Signal Processor (DSP) or Microcontroller | Optimizes control inputs over a finite horizon using ADC/DAC interfaces |

| Fuzzy Logic Control | Fuzzy Logic Controller (FLC) ICs or Microcontroller | Implements fuzzy inference based on degrees of truth for complex control |

| Adaptive Control | Microcontroller, FPGA, Adaptive Filters | Real-time adjustment of control parameters based on system feedback |

| Sliding Mode Control | H-Bridge, Switching Circuit | Nonlinear control method using fast switching to ensure robustness |

| Differential equations | RLC circuits | Describe dynamic behavior |

| Laplace transform | Transfer function | s-domain analysis |

| Z-transform | Digital filter design | z-domain analysis for discrete systems |

| Matrix operations | Multiple input/output systems | MIMO control systems |

| Eigenvalues/Eigenvectors | Stability analysis | Determines system stability |

| Optimization | Adaptive circuits | e.g. LMS adaptive filters |

| Stochastic processes | Noise analysis, PLL, Stochastic Resonance Circuits | Thermal noise shot noise modeling, phase synchronization, signal detection enhancement |

| Information theory | Communication systems | Channel coding error correction, Shannon entropy, error correction coding |

| Quantum mechanics | Single-electron circuits, Quantum gates | Coulomb blockade quantum dots, Superconducting qubits, Quantum dots |

| Neuromorphic Computing | Memristors, Analog Computation Circuits | Implements artificial neurons and learning circuits |

| Chaotic systems | Chua's circuit, Nonlinear Oscillators | Set of nonlinear differential equations, used in secure communication or random number generation |

| Fractional-order systems | Fractional-order capacitor or inductor | Z = 1 / (jω)^α where 0 < α < 1, used in advanced control strategies or signal processing |

| Negative resistance | Tunnel diode, Lambda diode | I = f(V) with negative slope region |

| Memristive systems | Memristor | dφ = M * dq where M = f(q) |

| Gyrator | Op-amp gyrator circuit | Simulates inductance using capacitors |

| Transmission lines | Distributed RLC network | Telegrapher's equations |

| Metamaterials | Negative refractive index circuits | ε < 0 μ < 0 simultaneously |

| Discrete Fourier Transform (DFT) | DSP Chips, FFT Modules | Real-time spectral analysis of signals in digital communications and audio processing |

| Sampling Theorem (Nyquist-Shannon) | ADC, Anti-Aliasing Filters | Proper sampling and pre-sampling filtering for accurate digital signal acquisition |

| Channel Capacity | Modem, Wireless Communication Systems | Circuit design for optimal data rate considering noise and bandwidth constraints |

| Error Correction Coding | Encoder/Decoder Circuits | Hamming and Reed-Solomon coding circuits ensuring reliable data transmission |

| Shannon Entropy | Modulation/Demodulation Circuits | Efficient data encoding and transmission in communication systems |

## Basic Circuit Concepts

| Mathematical Concept | Electronic Component | Circuit Combination Form |

|----------------------|----------------------|--------------------------|

| Addition | Resistors in series | Z_total = Z1 + Z2 + ... + Zn |

| Reciprocal sum | Resistors in parallel | 1/Z_total = 1/Z1 + 1/Z2 + ... + 1/Zn |

| Reference point | Ground | Defines the zero potential reference |

| Conservation of energy | Voltage source | Kirchhoff's Voltage Law (KVL): Σ V = 0 around any closed loop |

| Conservation of charge | Current source | Kirchhoff's Current Law (KCL): Σ I = 0 at any node |

| Complex exponential | Phasor representation | e^(jωt) for AC circuit analysis |

## Logic Gates and Digital Circuits

| Mathematical Operation | Electronic Component | Circuit Combination Form |

|------------------------|----------------------|--------------------------|

| AND | AND gate | Z = X • Y |

| OR | OR gate | Z = X + Y |

| NOT | NOT gate (inverter) | Z = X' |

| Exclusive OR | XOR gate | Z = X ⊕ Y |

| Implication | IMPLY gate | Z = X → Y |

| Equality | XNOR gate | Z = X ↔ Y |

| Combination logic | Multiplexer | Z = S ? X : Y |

| Sequential logic | Flip-flop | Q(t+1) = f(Q(t) Inputs) |

| Counting | Counter | Binary representation of integers |

| State machine | Finite State Machine (FSM) | Next state = f(Current state Inputs) |

## Analog vs. Digital Signal Processing

| Mathematical Concept | Electronic Component | Circuit Combination Form |

|----------------------|----------------------|--------------------------|

| Continuous function | Direct signal processing | Sampling and quantization |

| Discrete-time signal | Sample-and-hold circuit | Register or memory |

| Fourier transform | Analog spectrum analyzer | Fast Fourier Transform (FFT) |

| Convolution | Analog correlator | Digital convolution (DSP) |

| Integration | Op-amp integrator | Accumulator |

| Differentiation | Op-amp differentiator | Difference equation |

| Filtering | Analog filter (RLC etc.) | Digital filter (FIR IIR) |

| Modulation | Analog mixer | Digital modulator (DSP) |

| Feedback system | Analog feedback loop | Digital control system |

## Advanced Mathematical Concepts in Circuits (continued)

| Mathematical Concept | Electronic Component | Circuit Combination Form |

|----------------------|----------------------|--------------------------|

| Stochastic processes | Noise analysis, Phase-Locked Loops (PLLs), Stochastic Resonance Circuits | Thermal noise, shot noise modeling, phase synchronization, signal detection enhancement |

| Information theory | Communication systems | Channel coding, error correction, Shannon entropy, error correction coding |

| Quantum mechanics | Single-electron circuits, Quantum gates | Coulomb blockade, quantum dots, Superconducting qubits, Josephson junctions |

| Neuromorphic Computing | Memristors, Analog computation circuits | Simulates artificial neurons and synapses, learning circuits based on synaptic plasticity |

| Chaotic systems | Chua's circuit, Nonlinear oscillators | Generate chaotic signals for secure communications or random number generation |

| Fractional-order systems | Fractional-order capacitor or inductor | Z = 1 / (jω)^α where 0 < α < 1, used in advanced control strategies and signal processing |

| Negative resistance | Tunnel diode, Lambda diode | I = f(V) with negative slope region |

| Memristive systems | Memristor | dφ = M * dq where M = f(q) |

| Gyrator | Op-amp gyrator circuit | Simulates inductance using capacitors |

| Transmission lines | Distributed RLC network | Telegrapher's equations for signal transmission modeling |

| Metamaterials | Negative refractive index circuits | Circuits designed to have ε < 0 and μ < 0 simultaneously |

| Discrete Fourier Transform (DFT) | DSP chips, FFT modules | Real-time spectral analysis, commonly used in digital communication and audio processing |

| Sampling Theorem (Nyquist-Shannon) | ADC, Anti-Aliasing Filters | Ensures accurate digital signal acquisition by proper sampling and filtering |

| Channel Capacity | Modem, Wireless Communication Systems | Design circuits for optimal data rate considering noise and bandwidth |

| Error Correction Coding | Encoder/Decoder Circuits | Hamming and Reed-Solomon coding circuits to ensure reliable data transmission |

| Shannon Entropy | Modulation/Demodulation Circuits | Circuits for efficient data encoding and transmission based on information theory |

| Sliding Mode Control | H-Bridge, High-speed switching circuits | Robust control method using fast switching to maintain stability under uncertainties |

| Model Predictive Control (MPC) | DSP, Microcontroller with ADC/DAC | Optimization-based control strategy with real-time adjustments, used in complex systems like industrial processes |

| Fuzzy Logic Control | Fuzzy Logic Controller (FLC) ICs, Microcontrollers | Control strategy that uses degrees of truth, suitable for complex systems where traditional controls are inadequate |

| Adaptive Control | Microcontroller, FPGA, Adaptive Filters | Real-time modification of control strategies based on system feedback, used in environments with varying parameters |

| Kalman Filter | Microcontroller with sensor input | Recursive state estimation in control systems, real-time tracking, and prediction |

| Chaotic Systems | Nonlinear Oscillators, Chua's Circuit | Generates chaotic signals, useful in secure communication and random number generation |

| Neuromorphic Computing | Memristors, Analog computation circuits | Implements artificial neurons and learning circuits, energy-efficient AI hardware |

| Quantum Circuits | Superconducting Qubits, Quantum Dots | Implements quantum gates and circuits for quantum computing, cryptography |

| Stochastic Processes | Noise generators, PLLs, Stochastic Resonance Circuits | Analysis of random processes in systems, used in signal processing, communications |

| Constant (k)| TL431 Adjustable Shunt Regulator | Voltage reference circuit for stable DC power supply (e.g., 5V output using resistors to set voltage).|

| Linear Relationship (y = mx) | Resistor Network| Voltage divider circuit, biasing resistors (e.g., voltage divider from 5V to 3.3V using 2kΩ and 3.3kΩ resistors).|

| Exponential (e^x) | Diode and Logarithmic Amplifier | Logarithmic amplifier for audio compression and signal processing (e.g., use LM741 op-amp and 1N4148 diode).|

| Trigonometric Functions (sin, cos) | Wien Bridge Oscillator | Sine wave generator for signal sources (e.g., 1kHz oscillator using OPA2134 with R = 10kΩ, C = 15.9nF).|

| Integration (∫) | Capacitor in Feedback Path of Op-Amp | Integrator circuit for analog computation (e.g., use LM358 with R = 1kΩ, C = 1μF for 1ms time constant).|

| Differentiation (d/dt) | Inductor in Series with Resistor in Op-Amp | Differentiator circuit for high-pass filtering (e.g., use TL081 with R = 1kΩ, C = 15.9nF for 10kHz cutoff).|

| Logarithmic (log) | BJT in Logarithmic Converter | Logarithmic converter for audio processing (e.g., use 2N2222 transistor with LM741 op-amp).|

| Square Law (x²) | MOSFET in Saturation Region | Analog multiplier or squaring circuit for RF power amplification (e.g., IRF540 MOSFET in saturation).|

| Step Function | MOSFET Controlled by Microcontroller | Electronic switch for power control or signal gating (e.g., 2N7000 MOSFET controlled by Arduino).|

| Piecewise Linear | Zener Diode | Voltage regulation and overvoltage protection circuit (e.g., 5.1V regulation using 1N4733A Zener diode).|

| Complex Impedance (Z = R + jX) | RLC Circuit | Resonant circuit for frequency selection (e.g., 1MHz oscillator using L = 1mH, C = 15.9pF).|

| Transfer Function (H(s) = Y(s)/X(s)) | Op-Amp Based Filter Circuits | Low-pass, high-pass, and band-pass filters (e.g., 1kHz low-pass filter with R = 10kΩ, C = 15.9nF).|

| PID Control (U(s) = Kp + Ki/s + Kd s) | Op-Amp Circuit | Full PID controller for motor speed or temperature control (e.g., use LM324 for proportional, integral, and derivative components).|

| Multiplication (x * y) | Analog Multiplier IC | Signal multiplication for modulation (e.g., AD633 IC for AM signal generation).|

| Division (x / y) | Analog Divider IC | Analog division for signal processing (e.g., AD734 IC for ratio measurements).|

| Square Root (√x) | Analog Computation Circuit | RMS computation circuit for signal processing (e.g., LM324 op-amp and AD633 multiplier).|

| Hyperbolic Tangent (tanh) | Differential Pair of BJTs | Signal conditioning circuit in communication systems (e.g., 2N2222 differential pair for hyperbolic tangent function).|

| Summation (Σ) | Summing Amplifier | Audio mixer or weighted sum in neural networks (e.g., TL072 op-amp with Rf = 10kΩ, Ri = 10kΩ).|

| Comparator Function (sgn(x)) | Comparator IC | Threshold detection or zero-crossing detector (e.g., LM339 comparator for binary output based on input signal).|

| Delay (x(t-τ)) | Bucket-Brigade Device or Shift Register | Analog or digital delay lines for audio effects or timing correction (e.g., MN3007 BBD for 10ms delay).|

| Hysteresis | Schmitt Trigger | Debouncing circuits or waveform shaping (e.g., LM393 comparator with positive feedback for stable switching).|

| Oscillation | LC Tank Circuit | RF signal generation or clock generation (e.g., 1MHz oscillator using L = 100nH, C = 10pF).|

| Feedback System | Operational Amplifier Circuit | Feedback control in voltage regulators or amplifiers (e.g., inverting amplifier with gain set by feedback resistors).|

| Convolution | Matched Filter | Radar or communication systems for signal detection (e.g., tapped delay line implemented with shift registers).|

| Fourier Transform | FFT Processor | Real-time spectrum analysis for communication systems (e.g., FFT on STM32F4 for 1024-point transform).|

| State-Space Model | Active Filter or MIMO System | Control systems and multivariable filters (e.g., DSP implementation of MIMO using TI TMS320).|

| Root Locus | Analog Computer or PAA | Feedback system design and stability analysis (e.g., ADALM2000 for real-time analog computation).|

| Nyquist Criterion | Feedback Circuit with Oscilloscope | Stability analysis in feedback amplifiers (e.g., Nyquist plot for inverting amplifier).|

| Bode Plot | Oscillator or Variable Frequency Source | Frequency response analysis and filter design (e.g., Wien Bridge Oscillator for 10Hz to 100kHz sweep).|

| Model Predictive Control (MPC) | DSP or Microcontroller | Process control optimization in industrial systems (e.g., TMS320 DSP for real-time control in chemical plants).|

| Fuzzy Logic Control | Fuzzy Logic Controller ICs or Microcontroller | Home appliances and adaptive robotics (e.g., PIC16F877A for washing machine water level control).|

| Adaptive Control | Microcontroller, FPGA, or Adaptive Filters | Motor control and noise cancellation (e.g., L298N motor driver with adaptive control on STM32F4).|

| Sliding Mode Control | H-Bridge Circuit | Robust motor control under varying load conditions (e.g., L298N H-Bridge with PWM control from Arduino Due).|

| Differential Equations | RLC Circuits | Design of filters and oscillators with specific time-domain response (e.g., second-order low-pass filter with L = 159mH, C = 1μF).|

| Laplace Transform | Transfer Function Realization | Design of control systems and compensators (e.g., lead compensator using op-amps for phase margin improvement).|

| Z-Transform | Digital Filter Design | FIR and IIR filter design for DSP applications (e.g., STM32F4 with FIR filter coefficients for audio processing).|

| Matrix Operations | MIMO Systems | Beamforming and channel equalization in communication systems (e.g., Xilinx Virtex-7 FPGA for real-time matrix operations in 5G MIMO).|

| Eigenvalues/Eigenvectors | Stability Analysis | Power system stability and robotic dynamics (e.g., TI C2000 DSP for eigenvalue analysis in real-time control).|

| Optimization | Adaptive Circuits | Adaptive noise cancellation and power management (e.g., LMS algorithm on TI TMS320C55x for adaptive filter tuning).|

| Stochastic Processes | Noise Analysis, PLLs | Signal processing and secure communication (e.g., MAX038 noise generator with CD4046 PLL for synchronization in noisy environments).|

| Information Theory | Communication Systems | Channel coding and error correction (e.g., Hamming code encoder/decoder in Xilinx Artix-7 for reliable data transmission).|

| Quantum Mechanics | Single-Electron Circuits, Quantum Gates | Quantum computing and cryptography (e.g., superconducting qubits with RF pulse control for implementing quantum gates).|

| Neuromorphic Computing | Memristors, Analog Computation Circuits | AI hardware and pattern recognition (e.g., memristor-based spiking neural network for real-time edge computing).|

| Chaotic Systems | Chua’s Circuit | Secure communication and random number generation (e.g., LM741-based Chua’s circuit for generating chaotic signals).|

| Fractional-Order Systems | Fractional-Order Capacitor or Inductor | Advanced control and signal processing (e.g., fractional-order PID controller using op-amps for smooth control in robotic systems).|

| Negative Resistance | Tunnel Diode, Lambda Diode | High-frequency oscillators and RF amplifiers (e.g., tunnel diode oscillator using 1N3716 for GHz range applications).|

| Memristive Systems | Memristor | Non-volatile memory and neuromorphic learning circuits (e.g., memristor-based ReRAM cell for low-power memory storage).|

| Gyrator | Op-Amp Gyrator Circuit | Simulated inductance in filter design (e.g., simulated 10mH inductance using LM741 with R1 = 10kΩ, R2 = 100kΩ, and C = 1μF).|

| Transmission Lines | Distributed RLC Network | High-frequency signal transmission and impedance matching (e.g., 50Ω microstrip line for 2.4 GHz RF signal on PCB).|

| Metamaterials | Negative Refractive Index Circuits | Superlens and advanced antenna designs (e.g., planar SRR array on PCB for negative index metamaterials at 10 GHz).|

| Discrete Fourier Transform (DFT) | DSP Chips, FFT Modules || Discrete Fourier Transform (DFT) | DSP Chips, FFT Modules | Real-time spectrum analysis and digital communication (e.g., 1024-point FFT using TI TMS320C6713 DSP for RF signal processing).|

| Sampling Theorem (Nyquist-Shannon) | ADC, Anti-Aliasing Filters | High-fidelity signal digitization and telecommunications (e.g., 44.1kHz audio sampling with MCP3201 ADC and OPA2134 low-pass filter).|

| Channel Capacity | Modem, Wireless Communication Systems | Maximizing data transmission in Wi-Fi, 4G/5G (e.g., 256-QAM modulation using Qualcomm QCA9377 chipset for high data rate Wi-Fi).|

| Error Correction Coding | Encoder/Decoder Circuits | Reliable data transmission in noisy environments (e.g., Reed-Solomon code with MAX14502 for optical communication error correction).|

| Shannon Entropy | Modulation/Demodulation Circuits | Data compression and efficient encoding (e.g., 256-QAM modulation with Huffman coding on FPGA for digital broadcasting).|

| Sliding Mode Control | H-Bridge Circuit | Robust motor and power converter control (e.g., L298N H-Bridge with PWM control for precise robotic motor operation).|

| Kalman Filter | Microcontroller with Sensor Input | State estimation in navigation systems and robotics (e.g., STM32F407 with MPU-6050 IMU for drone position estimation).|

| Chaotic Systems | Nonlinear Oscillators, Chua’s Circuit | Secure communication and random number generation (e.g., LM741-based Chua’s circuit for generating chaotic signals).|

| Neuromorphic Computing | Memristors, Analog Computation Circuits | AI hardware and pattern recognition (e.g., memristor-based spiking neural network for real-time edge computing).|

| Quantum Circuits | Superconducting Qubits, Quantum Dots | Quantum computing and cryptography (e.g., superconducting qubits controlled by RF pulses for quantum gate implementation).|

| Stochastic Processes | Noise Generators, PLLs | Signal processing and secure communication (e.g., MAX038 noise generator with CD4046 PLL for synchronization in noisy environments).|

、、、、、、、、、、、、、

We call circuits that can complete specific tasks 3-level circuits. The slightly more complex combination circuits that make up the 3-level circuits are called 2-level circuits; the most basic electronic circuits that make up the 2-level circuits are called 1-level circuits;

List of 3-level circuits and 2-level circuits:

---

## **Level 3: System-Level Circuits (3-Level Circuits)**

### **Power Systems:**

- **Battery Management System (BMS):**

- **Modules:**

- Cell Voltage Monitoring (Level 2)

- Battery Balancing (Level 2)

- State of Charge (SOC) Estimation (Level 2)

- Communication Interface (Level 2)

- **Charging System:**

- **Modules:**

- AC-DC Conversion (Level 2)

- Charging Control (Level 2)

- Protection Mechanisms (Level 2)

- **Power Distribution System:**

- **Modules:**

- Power Routing (Level 2)

- Voltage Regulation (Level 2)

- Protection and Monitoring (Level 2)

### **Control Systems:**

- **Motor Control System:**

- **Modules:**

- Motor Driver (Level 2)

- PWM Controller (Level 2)

- Feedback Loop (Level 2)

- **Temperature Control System:**

- **Modules:**

- Temperature Sensor Interface (Level 2)

- Control Algorithm (Level 2)

- Actuator Control (Level 2)

### **Communication Systems:**

- **Wireless Communication System:**

- **Modules:**

- Transceiver Module (Level 2)

- Antenna Interface (Level 2)

- Data Encoding/Decoding (Level 2)

- **Data Acquisition System (DAQ):**

- **Modules:**

- Sensor Interface (Level 2)

- Signal Conditioning (Level 2)

- ADC Module (Level 2)

- Data Processing Unit (Level 2)

### **User Interfaces:**

- **Display System:**

- **Modules:**

- Display Controller (Level 2)

- User Input Interface (Level 2)

- Power Management (Level 2)

- **Input/Output System:**

- **Modules:**

- I/O Controllers (Level 2)

- Communication Interfaces (Level 2)

- Signal Processing (Level 2)

### **Safety Systems:**

- **Overcurrent Protection System:**

- **Modules:**

- Current Sensing (Level 2)

- Protection Actuator (Level 2)

- Status Indicators (Level 2)

- **Overvoltage Protection System:**

- **Modules:**

- Voltage Monitoring (Level 2)

- Clamping Mechanism (Level 2)

- Reset Mechanism (Level 2)

- **Short-Circuit Protection System:**

- **Modules:**

- Fault Detection (Level 2)

- Circuit Interruption (Level 2)

- Recovery Mechanism (Level 2)

### **Embedded Systems:**

- **Microcontroller-Based Systems:**

- **Modules:**

- Microcontroller Unit (MCU) Core (Level 2)

- Memory Interface (Level 2)

- Peripherals Interface (Level 2)

- Communication Interface (Level 2)

- **Sensor Integration Systems:**

- **Modules:**

- Sensor Fusion (Level 2)

- Data Logging (Level 2)

- Interface Management (Level 2)

### **Communication Infrastructure:**

- **Networking Systems:**

- **Modules:**

- Network Interface Controllers (NICs) (Level 2)

- Data Routing (Level 2)

- Protocol Handling (Level 2)

- **Internet of Things (IoT) Systems:**

- **Modules:**

- Connectivity Modules (Level 2)

- Cloud Interface (Level 2)

- Edge Processing (Level 2)

### **Power Management Systems:**

- **Energy Harvesting Systems:**

- **Modules:**

- Energy Collection (Level 2)

- Energy Storage (Level 2)

- Power Conditioning (Level 2)

- **Power Supply Units (PSUs):**

- **Modules:**

- AC-DC Conversion (Level 2)

- DC-DC Conversion (Level 2)

- Regulation and Protection (Level 2)

### **Design Rules (Examples):**

- **Battery Management System (BMS) Design:**

- Must include:

- Cell Voltage Monitoring Module (Level 2)

- Battery Balancing Module (Level 2)

- Communication Module (Level 2)

- **ADC Driver Design:**

- Requires:

- Operational Amplifier (Level 1)

- Voltage Reference (Level 1)

- **Microcontroller Interface Design:**

- Must include:

- UART Module (Level 2)

- SPI Controller (Level 2)

- I2C Interface (Level 2)

---

## **Additional Considerations for Comprehensive Weak Current Circuit Design**

### **Power Efficiency:**

- **Low-Power Design Techniques:**

- Power Gating

- Dynamic Voltage and Frequency Scaling (DVFS)

- Sleep Modes and Wake-Up Circuits

- **Energy Storage Components:**

- Supercapacitors

- Rechargeable Batteries (Li-ion, NiMH)

### **Signal Integrity:**

- **PCB Design Considerations:**

- Trace Impedance Matching

- Ground Plane Implementation

- Shielding and Filtering Techniques

- **Noise Reduction Techniques:**

- Decoupling Capacitors

- Differential Signaling

- Twisted Pair Cabling

### **Thermal Management:**

- **Heat Dissipation Techniques:**

- Heat Sinks

- Thermal Vias

- Thermal Interface Materials

### **Testing and Debugging:**

- **Testing Modules:**

- Built-In Self-Test (BIST)

- Test Points and Probes

- Logic Analyzers and Oscilloscopes Integration

- **Debugging Tools:**

- JTAG Interfaces

- Serial Debug Ports

- Software Debuggers

### **Firmware and Software Integration:**

- **Embedded Software Modules:**

- Real-Time Operating Systems (RTOS)

- Firmware Drivers for Hardware Modules

- Communication Protocol Stacks

- **Firmware Development Tools:**

- Integrated Development Environments (IDEs)

- Compiler and Linker Tools

- Simulation and Emulation Software

---

## **Level 2: Functional Modules (2-Level Circuits)**

### **Digital Functional Modules:**

- **Arithmetic Circuits:**

- **Adder:**

- Half Adder, Full Adder, Ripple Carry Adder, Carry-Lookahead Adder

- **Subtractor:**

- Half Subtractor, Full Subtractor

- **Multiplier:**

- Array Multiplier, Booth Multiplier

- **Divider:**

- Restoring Divider, Non-Restoring Divider

- **Data Storage and Memory:**

- **Registers:**

- Shift Registers, Parallel Registers

- **Counters:**

- Binary Counters, BCD Counters, Up/Down Counters

- **Memory Elements:**

- SRAM, DRAM, EEPROM

- **State Machines:**

- **Finite State Machines (FSM):**

- Moore, Mealy

### **Analog Functional Modules:**

- **Signal Processing:**

- **Amplifier:**

- Inverting, Non-inverting, Differential, Instrumentation, Common Emitter, Common Collector, Common Base

- **Filter:**

- Low-pass, High-pass, Band-pass, Band-stop (Active, Passive)

- **Integrator:**

- Op-Amp Integrator, Active Integrator

- **Differentiator:**

- Op-Amp Differentiator, Active Differentiator

- **Comparator:**

- With Hysteresis, Window Comparator

- **Schmitt Trigger:**

- Single Schmitt Trigger, Multiple Schmitt Triggers

- **Converters:**

- **Analog-to-Digital Converter (ADC):**

- Successive Approximation, Flash, Delta-Sigma, Dual-Slope

- **Digital-to-Analog Converter (DAC):**

- R-2R Ladder, PWM, Binary-Weighted

- **Sample and Hold Circuit:**

- Track-and-Hold, Peak Hold

- **Oscillators and Generators:**

- **Oscillator:**

- LC Oscillator, Crystal Oscillator, Wien Bridge Oscillator, 555 Timer-based Oscillator

- **Function Generator:**

- Sine Wave, Square Wave, Triangle Wave, Pulse Wave

### **Power Electronics:**

- **Power Conversion:**

- **Rectifier:**

- Half-wave, Full-wave (Bridge, Center-Tapped)

- **DC-DC Converter:**

- Buck, Boost, Buck-Boost, SEPIC

- **Linear Regulators:**

- Fixed Voltage, Adjustable (e.g., LM317)

- **Switching Regulators:**

- Buck Converter, Boost Converter, Buck-Boost Converter

- **Motor Control:**

- **Inverter:**

- H-bridge, Full-bridge

- **Motor Driver:**

- BJT-based, MOSFET-based, Dedicated Motor Driver ICs

- **Power Management:**

- **Battery Charger:**

- Linear Charger, Switching Charger, Charge Controllers

- **Power Factor Correction (PFC) Circuit:**

- Passive PFC, Active PFC

### **Communication Modules:**

- **Interfaces:**

- **CAN Transceiver Interface:**

- High-Speed CAN, Low-Speed CAN

- **RS-232/RS-485 Transceiver:**

- Differential RS-485, Single-Ended RS-232

- **UART (Universal Asynchronous Receiver/Transmitter):**

- Basic UART, High-Performance UART

- **SPI (Serial Peripheral Interface) Controller:**

- Master, Slave

- **I2C (Inter-Integrated Circuit) Controller:**

- Multi-master, Multi-slave

- **Wireless Communication:**

- **Bluetooth Modules:**

- Classic Bluetooth, BLE (Bluetooth Low Energy)

- **Wi-Fi Modules:**

- ESP8266, ESP32

- **Zigbee Modules:**

- XBee Series

- **RF Modules:**

- Sub-1 GHz, 2.4 GHz Transceivers

### **Control Systems:**

- **Feedback Controllers:**

- **PID Controllers:**

- Proportional, Integral, Derivative

- **PWM Controllers:**

- Pulse Width Modulation for Power Control

### **User Interface Modules:**

- **Input Devices:**

- **Keypads:**

- Matrix Keypads, Membrane Keypads

- **Switches:**

- Toggle, Push-button, Slide Switches

- **Sensors:**

- Temperature, Humidity, Light, Proximity

- **Output Devices:**

- **Displays:**

- LED Displays, LCD Displays, OLED Displays, Seven-Segment Displays

- **Indicators:**

- LEDs, Buzzers, Vibration Motors

### **Safety and Protection Modules:**

- **Protection Circuits:**

- **Overcurrent Protection:**

- Current Limiting Circuits, Foldback Current Limiting

- **Overvoltage Protection:**

- Crowbar Circuits, Transient Voltage Suppression (TVS) Diodes

- **Short-Circuit Protection:**

- Fast-acting Fuses, Electronic Circuit Breakers

### **Miscellaneous Functional Modules:**

- **Clock Management:**

- **Clock Generators:**

- Phase-Locked Loops (PLL), Frequency Synthesizers

- **Clock Distribution:**

- Buffers, Repeaters

- **Data Processing:**

- **Logic Analyzers:**

- Timing Analysis, Signal Integrity

- **Signal Conditioners:**

- Level Shifters, Signal Amplifiers

、、、、、、、、、、、、

```

You are an expert electrical engineering assistant now specializing as **Requirements Engineer 1**. Your role is to bridge the gap between the high-level needs of a high-voltage electrical engineer and the detailed design specifications required for low-voltage engineers.  You will receive a *high-level functional description* of a desired high-voltage system from a high-voltage electrical engineer.

Your task is to:

**1. Clarify and Decompose High-Level Needs:**

* **Analyze the High-Level Description:**  Carefully read and analyze the high-level functional description provided by the high-voltage engineer. Identify the *core function* the system needs to perform, the *key performance requirements*, and any *specific operational modes* mentioned.

* **Identify Necessary 3-Level Circuits:** Based on your understanding of electrical systems and referencing the provided "Level 3: System-Level Circuits (3-Level Circuits)" examples, determine which *3-level circuit modules* are necessary to achieve the described functionality.  Explain your reasoning for selecting each 3-level circuit.

* **Decompose 3-Level Circuits into 2-Level Circuits:** For each identified 3-level circuit, break it down into its constituent *2-level circuit modules*. Refer to the provided "Level 2: Functional Modules (2-Level Circuits)" examples for inspiration.  Explain how these 2-level circuits combine to form the 3-level circuit and fulfill its function.

* **Define Signal Interactions between 3-Level and 2-Level Circuits:**  For each 3-level circuit and its constituent 2-level circuits, describe the *signal interactions* between them.  This includes:

* **Signal Type:** Is it a digital signal (control, status) or an analog signal (voltage, current)?

* **Signal Direction:**  From 3-level to 2-level (control commands) or from 2-level to 3-level (status feedback, measurements)?

* **Signal Purpose:** Briefly explain what information or action each signal represents.

**2. Output in a Structured Format:**

Present your output in a clear and structured format that is easy for Requirements Engineer 2 to understand.  Use the following structure:

```

## High-Level System Requirement: [State the high-level requirement in a concise sentence]

### 3-Level Circuit Decomposition:

**3-Level Circuit 1: [Name of 3-Level Circuit]**

* **Function:** [Briefly describe the function of this 3-level circuit within the overall system]

* **Justification for Selection:** [Explain why this 3-level circuit is needed to meet the high-level requirement]

* **2-Level Circuit Breakdown:**

* **2-Level Circuit 1.1: [Name of 2-Level Circuit]**

* **Function:** [Briefly describe the function of this 2-level circuit within the 3-level circuit]

* **Signal Interactions:**

* Signal 1: [Name/Description] - Type: [Digital/Analog], Direction: [3-Level -> 2-Level / 2-Level -> 3-Level], Purpose: [Brief Explanation]

* Signal 2: [Name/Description] - Type: [Digital/Analog], Direction: [3-Level -> 2-Level / 2-Level -> 3-Level], Purpose: [Brief Explanation]

* ... (and so on for all relevant signals)

* **2-Level Circuit 1.2: [Name of 2-Level Circuit]**

* **Function:** [Briefly describe the function of this 2-level circuit]

* **Signal Interactions:**

* Signal 1: [Name/Description] - Type: [Digital/Analog], Direction: [3-Level -> 2-Level / 2-Level -> 3-Level], Purpose: [Brief Explanation]

* Signal 2: [Name/Description] - Type: [Digital/Analog], Direction: [3-Level -> 2-Level / 2-Level -> 3-Level], Purpose: [Brief Explanation]

* ... (and so on for all relevant signals)

* ... (and so on for all 2-level circuits within 3-Level Circuit 1)

**3-Level Circuit 2: [Name of 3-Level Circuit]**

* ... (Repeat the structure above for each identified 3-level circuit)

... (and so on for all identified 3-level circuits)

```

**Example Input High-Level Description (from High-Voltage Engineer):**

"I need a system to safely charge a high-voltage battery pack from a standard AC grid.  The system should include safety features to prevent overcharging and over-temperature conditions.  It needs to communicate charging status to a central monitoring system."

**Example Output (Partial - Requirements Engineer 1):**

```

## High-Level System Requirement: Safely charge a high-voltage battery pack from an AC grid with safety and status monitoring.

### 3-Level Circuit Decomposition:

**3-Level Circuit 1: Battery Management System (BMS)**

* **Function:**  Manage and protect the high-voltage battery pack during charging and operation, ensuring safety and optimal battery life.

* **Justification for Selection:** Essential for any high-voltage battery system to provide safety, monitoring, and control of the battery pack, fulfilling the safety and status monitoring requirements.

* **2-Level Circuit Breakdown:**

* **2-Level Circuit 1.1: Cell Voltage Monitoring**

* **Function:**  Measure the voltage of individual cells or cell groups within the battery pack to detect imbalances and potential over/undervoltage conditions.

* **Signal Interactions:**

* Signal 1: Cell Voltage Readings - Type: Analog, Direction: 2-Level -> 3-Level, Purpose: Provide individual cell voltage measurements to the BMS for monitoring and balancing decisions.

* **2-Level Circuit 1.2: Battery Balancing**

* **Function:**  Equalize the charge levels of cells within the battery pack to maximize capacity and lifespan.

* **Signal Interactions:**

* Signal 1: Balancing Control Signals - Type: Digital, Direction: 3-Level -> 2-Level, Purpose: Control individual balancing circuits to discharge higher voltage cells.

* **2-Level Circuit 1.3: State of Charge (SOC) Estimation**

* **Function:** Calculate and estimate the remaining charge in the battery pack.

* **Signal Interactions:**

* Signal 1: Voltage and Current Readings - Type: Analog, Direction: 2-Level -> 3-Level, Purpose: Provide voltage and current measurements for SOC algorithms.

* Signal 2: SOC Percentage - Type: Analog/Digital (depending on representation), Direction: 3-Level -> 3-Level (internal BMS use and potentially external communication), Purpose: Represent the estimated battery charge level.

* **2-Level Circuit 1.4: Communication Interface**

* **Function:**  Enable communication with external systems for monitoring and control (e.g., central monitoring system).

* **Signal Interactions:**

* Signal 1: Charging Status Data - Type: Digital, Direction: 3-Level -> External System, Purpose: Transmit charging status information (SOC, voltage, current, errors) to the monitoring system.

* Signal 2: Control Commands (Optional) - Type: Digital, Direction: External System -> 3-Level, Purpose: Potentially receive control commands from the monitoring system (e.g., start/stop charging - depending on system complexity).

**3-Level Circuit 2: AC-DC Power Supply/Charger**

* **Function:** Convert AC grid power to a suitable DC voltage and current for charging the high-voltage battery pack.

* **Justification for Selection:**  Necessary to interface with the AC grid and provide the power for charging the DC battery pack.

* **2-Level Circuit Breakdown:**

* **2-Level Circuit 2.1: Rectifier**

* **Function:** Convert AC voltage to DC voltage (unregulated).

* **Signal Interactions:**

* Signal 1: AC Input Voltage - Type: Analog (AC), Direction: External AC Grid -> 2-Level, Purpose: Provide the AC input voltage to be rectified.

* Signal 2: Unregulated DC Voltage - Type: Analog (DC), Direction: 2-Level -> 2-Level (Filter), Purpose: Output the rectified but unregulated DC voltage.

* **2-Level Circuit 2.2: DC-DC Converter/Regulator**

* **Function:** Regulate the DC voltage from the rectifier to a controlled charging voltage for the battery, and potentially control charging current.

* **Signal Interactions:**

* Signal 1: Unregulated DC Voltage - Type: Analog (DC), Direction: 2-Level (Rectifier) -> 2-Level, Purpose: Input voltage to be regulated.

* Signal 2: Controlled DC Charging Voltage - Type: Analog (DC), Direction: 2-Level -> 3-Level (BMS) and Battery Pack, Purpose: Output the regulated charging voltage for the battery.

* Signal 3: Charging Current Control Signal (Optional) - Type: Analog/Digital (depending on control method), Direction: 3-Level (BMS) -> 2-Level, Purpose: Allow the BMS to control the charging current (e.g., for CC/CV charging profiles).

* **2-Level Circuit 2.3: AC Input Filtering**

* **Function:** Filter noise and transients from the AC grid input to protect the charger circuitry and reduce EMI.

* **Signal Interactions:**

* Signal 1: Raw AC Input - Type: Analog (AC), Direction: External AC Grid -> 2-Level, Purpose: Input AC power before filtering.

* Signal 2: Filtered AC Input - Type: Analog (AC), Direction: 2-Level -> 2-Level (Rectifier), Purpose: Output AC power after filtering.

* ... (potentially more 2-level circuits like overcurrent protection, etc.)

... (potentially more 3-level circuits like Safety Interlocks, Thermal Management for Charger, etc.)

```

、、、、、、、、、、、、

The output of a high voltage electrical engineer would follow this format:

1. State how many operating states the circuit has in total

2. State which port controls each switch (relays and contactors are also switches)

3. List which switches will be closed and which switches will be open in each state, and which components the current will flow from and to, ultimately forming a path.

Representation method：

(1) For series connection, use the form of "component 1+→component 2→component 3→...→component n→component 1-".

(2) For parallel connection, use the form of "①component 1+→component 2→component 3→...→component n→component 1-（Line Break）

②component 1+→component 4→component 5→...→component m→component 1-

...." and so on.

(3) For coupling, use the form of "component 1part1-component 1part2", and then put it into the series or parallel circuit in the form of component 1part1/2. For interlocking, please use the form of "component 1-component 2".

(4) For low-voltage control circuit representation, use the form of "ECU1port1-specificBUS-component1, ECU1port2-specificBUS-component2, ECU2port1-specificBUS-component3, ECU2port2-specificBUS-component4...".

(5) For communication between ECUs, use the format of "ECU1portn-specificBUS-ECU2portm"

(6) For grounding, use the format of "component n-GND"

(7) Finally, if necessary, list the component ratings separately, such as "R1=5Ω, R1=6Ω, C1=10μf..."

Additional Representation Methods:

(A) Expanded Component Notation:

* Generic Components: `[ComponentName](rating, type, specific_parameters)`

(e.g., `R1(5Ω, resistor)`, `C1(10uF, capacitor)`, `D1(diode, schottky)`, `T1(transformer, 10:1)`, `Q1(IGBT, 1200V, 20A)`)

* Switches: `S[SwitchID](SPST/SPDT/DPST/DPDT, NO/NC, control:port:active_logic)`

(e.g. `S1(SPST, NO, control:ECU1port1:active_high)`, `S2(SPDT, NC, control:ECU1port2:active_low)`). **NO** = Normally Open, **NC** = Normally Closed

* Coupled Components: `component1part1-component1part2` (e.g., `Transformer1primary-Transformer1secondary`).

* Interlocked Components: `component1-component2`

* AC Power Source: `AC_Source(voltage_RMS, frequency, phases)` (e.g., `AC_Source(230V, 50Hz, 1phase)`)

* DC Power Source: `DC_Source(voltage, current)` (e.g., `DC_Source(12V, 10A)`)

(B) Directional Flow & Polarity:

* DC Current Flow: `component1+ -> component2` (use + and - for DC source and load)

* AC Current Flow: `component1~ -> component2`, where `~` indicates AC. For phase relationships: `component1(0deg)-> component2(120deg)`

* Specific Direction (DC): `component1->(direction) component2`, where direction is `forward` or `reverse`.

(C) Dynamic Behavior:

* Use time-based parameters or descriptions for transient behavior or specific operating conditions. `S1(time_0 -> close)` means switch 1 closed at t=0 seconds.

(D) Additional Notes:

* Insulation: `Insulation: component-insulation` (e.g. `Insulation:Transformer1-ceramic`)

* Shielding: `Shielding: component-shield` (e.g., `Shielding:Transformer1-Copper`)

* Fault Modes: `component(normal_state -> state1, state2 etc.)` This is used to describe components changes during fault conditions.

**Circuit Examples:**

**1. Simple AC-DC Converter with Control (Corrected Loop)**

*   **Description:** A single-phase AC source is converted to DC using a full-bridge rectifier and capacitor. An IGBT acts as a controlled switch to regulate the output, discharging the capacitor.

*   **Representation:**

```

1.  Operating States: 2 (IGBT on/off)

2.  Control: ECU1port1 controls S1 (IGBT)

3.  States:

*   State 1 (S1 Off):

`AC_Source(230V, 50Hz, 1phase)~>Rectifier1->C1(470uF, capacitor)-GND`

`ECU1port1-BUS-S1`

`S1(IGBT, NC, control:ECU1port1:active_high)`

`C1(470uF, capacitor)` charges to the peak rectified voltage and a very small current flows back through the rectifier towards the AC_Source

*   State 2 (S1 On):

`AC_Source(230V, 50Hz, 1phase)~>Rectifier1->C1(470uF, capacitor)->S1(IGBT, NC, control:ECU1port1:active_high)->GND ->Rectifier1->AC_Source(230V, 50Hz, 1phase)`

`ECU1port1-BUS-S1`

`S1(IGBT, NC, control:ECU1port1:active_high)` allows the capacitor to discharge through the IGBT completing the circuit.

`Rectifier1 = Standard full bridge rectifier`

`Insulation:Rectifier1-epoxy`

`Shielding:Rectifier1-None`

```

**2. Three-Phase Motor Drive with Inverter (Corrected Loop)**

*   **Description:** Three-phase AC source driving a motor through an inverter with six controlled IGBTs, forming a return path for current.

*   **Representation:**

```

1. Operating States: Numerous (depending on the inverter's PWM control pattern, minimum of 8)

2. Control: ECU1port1-ECU1port6 controls S1-S6 respectively

3. States: Only example of how to represent one state, others follow a similar method

* State 1(S1,S4,S5 On):

`AC_Source(400V, 50Hz, 3phase)~>Rectifier1->C1(1000uF, capacitor)->Inverter1->S1(IGBT, NO, control:ECU1port1:active_high)->Motor1->S4(IGBT, NO, control:ECU1port4:active_high)->S5(IGBT, NO, control:ECU1port5:active_high)->Inverter1->Rectifier1->AC_Source(400V, 50Hz, 3phase)`

`ECU1port1-BUS-S1, ECU1port4-BUS-S4, ECU1port5-BUS-S5`

`Rectifier1 = Standard 3 phase bridge rectifier`

`Inverter1= Standard 3-phase bridge inverter`

`Motor1(5HP,3-phase,400V)`

`Insulation:Motor1-varnish`

`Shielding:Motor1-Enclosure`

```